Name: Pranali Sonawane

Subject: CCNA

Lab – Multilayer Switch with DHCP:

First, we have taken one switch and five PCs. The PCs are connected to the switch using straight-through cables. We used a Multilayer Switch 0 (3560-24PS) because it is a Layer 3 switch.

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

!--------------------------

! Interface Fa0/1 - Network 10.0.0.0/8

!--------------------------

Switch(config)#interface fa0/1

Switch(config-if)#no switchport

Switch(config-if)#ip address 10.0.0.1 255.0.0.0

Switch(config-if)#no shutdown

Switch(config-if)#exit

! Create DHCP pool for 10.0.0.0 network

Switch(config)#ip dhcp pool DHCP1

Switch(dhcp-config)#network 10.0.0.0 255.0.0.0

Switch(dhcp-config)#default-router 10.0.0.1

Switch(dhcp-config)#exit

!--------------------------

! Interface Fa0/2 - Network 20.0.0.0/8

!--------------------------

Switch(config)#interface fa0/2

Switch(config-if)#no switchport

Switch(config-if)#ip address 20.0.0.1 255.0.0.0

Switch(config-if)#no shutdown

Switch(config-if)#exit

! Create DHCP pool for 20.0.0.0 network

Switch(config)#ip dhcp pool DHCP2

Switch(dhcp-config)#network 20.0.0.0 255.0.0.0

Switch(dhcp-config)#default-router 20.0.0.1

Switch(dhcp-config)#exit

!--------------------------

! Interface Fa0/3 - Network 30.0.0.0/8

!--------------------------

Switch(config)#interface fa0/3

Switch(config-if)#no switchport

Switch(config-if)#ip address 30.0.0.1 255.0.0.0

Switch(config-if)#no shutdown

Switch(config-if)#exit

! Create DHCP pool for 30.0.0.0 network

Switch(config)#ip dhcp pool DHCP3

Switch(dhcp-config)#network 30.0.0.0 255.0.0.0

Switch(dhcp-config)#default-router 30.0.0.1

Switch(dhcp-config)#exit

!--------------------------

! Interface Fa0/4 - Network 40.0.0.0/8

!--------------------------

Switch(config)#interface fa0/4

Switch(config-if)#no switchport

Switch(config-if)#ip address 40.0.0.1 255.0.0.0

Switch(config-if)#no shutdown

Switch(config-if)#exit

! Create DHCP pool for 40.0.0.0 network

Switch(config)#ip dhcp pool DHCP4

Switch(dhcp-config)#network 40.0.0.0 255.0.0.0

Switch(dhcp-config)#default-router 40.0.0.1

Switch(dhcp-config)#exit

!--------------------------

! Enable IP routing on the multilayer switch

!--------------------------

Switch(config)#ip routing