Name: Pranali Sonawane

Subject: CCNA

Lab – Port Security using static Configuration:

First, we have taken one switch and two PCs. The PCs are connected to the switch using straight-through cables. The PCs are configured using static IP addresses.

Switch (config)#interface fa0/1

Switch (config-if)#switchport

Switch (config-if)#switchport mode access

Switch (config-if)#switchport port-security

Switch (config-if)#switchport port-security mac-address sticky

Switch (config-if)#switchport port-security maximum 1

Switch (config-if)#switchport port-security violation shutdown

Switch (config-if)#do sh his

Switch (config-if)#exit

Switch (config)#exit

Switch#show port-security

After pinging the PCs, the current address count is 1. In the diagram above, when PC0 is moved from port Fa0/1 to PC5, it shows a red color (indicating a connection issue). After reconnecting PC0, enter the following commands:

Switch> enable

Switch# configure terminal

Switch(config)# interface fa0/1

Switch(config-if)# shutdown

Switch(config-if)# no shutdown