Name: Pranali Sonawane

Subject: CCNA

Lab – HSRP(Hot  standby Router Protocol):

First, we have taken one switch, two PCs, and two routers. The PCs are connected to the switch using straight-through cables. The routers are also connected to the switch using straight-through cables.

Configure PCs

- PC0:

  - IPv4 Address: 10.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.10

- PC1:

  - IPv4 Address: 10.0.0.4

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.10

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#standby ip 10.0.0.10

Router(config-if)#standby priority 101

Router(config-if)#standby pre-empt

Router#show standby brief

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.2  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#standby ip 10.0.0.10

Router(config-if)#standby priority 102

Router(config-if)#standby pre-empt

Router#show standby brief

Open pc 0 – cmd prompt – type command

>ping 10.0.0.10

Router 0 - Initial Configuration (Interface in Shutdown State)

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# shutdown

Router#show standby brief

First, we have taken one switch, two PCs, and five routers. The PCs are connected to the switch using straight-through cables. The routers are also connected to the switch using straight-through cables.

Configure PCs

- PC0:

  - IPv4 Address: 10.0.0.6

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.10

- PC1:

  - IPv4 Address: 10.0.0.7

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.10

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#standby ip 10.0.0.10

Router(config-if)#standby priority 101

Router(config-if)#standby pre-empt

Router#show standby brief

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.2  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#standby ip 10.0.0.10

Router(config-if)#standby priority 102

Router(config-if)#standby pre-empt

Router#show standby brief

Open Router 3 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.3  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#standby ip 10.0.0.10

Router(config-if)#standby priority 103

Router(config-if)#standby pre-empt

Router#show standby brief

Open Router 4 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.4  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#standby ip 10.0.0.10

Router(config-if)#standby priority 104

Router(config-if)#standby pre-empt

Router#show standby brief

Open Router 5 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.5  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#standby ip 10.0.0.10

Router(config-if)#standby priority 105

Router(config-if)#standby pre-empt

Router#show standby brief

Open pc 0 – cmd prompt – type command

>ping 10.0.0.10