Name: Pranali Sonawane

Subject: CCNA

Lab – HSRP(Hot  standby Router Protocol):

First, we have taken one switch, two PCs, and two routers. The PCs are connected to the switch using straight-through cables. The routers are also connected to the switch using straight-through cables.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#ip dhcp pool DHCP

Router(dhcp-config)#network 10.0.0.0 255.0.0.0

Router(dhcp-config)#default-router 10.0.0.1

Router(dhcp-config)# exit

Router(config)# interface fastEthernet 0/0

Router(config-if)#standby  ip 10.0.0.10

Router(config-if)#standby priority 110

Router(config-if)#standby  pre-empt

Router(dhcp-config)# exit

Router(config-if)#ip dhcp pool DHCP

Router(dhcp-config)#network 10.0.0.0 255.0.0.0

Router(dhcp-config)#default-router 10.0.0.1

Router(dhcp-config)# exit

Router#show standby brief

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.2  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#ip dhcp pool DHCP

Router(dhcp-config)#network 10.0.0.0 255.0.0.0

Router(dhcp-config)#default-router 10.0.0.1

Router(dhcp-config)# exit

Router(config)# interface fastEthernet 0/0

Router(config-if)#standby  ip 10.0.0.10

Router(config-if)#standby priority 120

Router(config-if)#standby  pre-empt

Router(dhcp-config)# exit

Router(config-if)#ip dhcp pool DHCP

Router(dhcp-config)#network 10.0.0.0 255.0.0.0

Router(dhcp-config)#default-router 10.0.0.10

Router(dhcp-config)# exit

Router#show standby brief

Open pc 0 – cmd prompt – type command>ping 10.0.0.10