Name: Pranali Sonawane

Subject: CCNA

Lab – HSRP(Hot  standby Router Protocol):

First, we have taken one switch, two PCs, and two routers. The PCs are connected to the switch using straight-through cables. The routers are also connected to the switch using straight-through cables.

Open Switch 0 – CLI prompt then type following commands

Switch>enable

Switch#config terminal

For VLAN 10 :

Switch (config)#vlan 10

Switch (config-vlan)# name CCNA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/1

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#exit

For VLAN 20 :

Switch (config)#vlan 20

Switch (config-vlan)#name MCSA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch(config-if)#exit

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch(config-if)#exit

Switch (config)#interface fastEthernet 0/4

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch(config-if)#exit

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.2  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#standby 1 ip 10.0.0.1

Router(config-if)#standby 1 priority 110

Router(config-if)#standby 1 pre-empt

Router#show standby brief

Router(config-if)#ip dhcp pool DHCP

Router(dhcp-config)#network 10.0.0.0 255.0.0.0

Router(dhcp-config)#default-router 10.0.0.1

Router(dhcp-config)#ip dhcp excluded-address 10.0.0.1 10.0.0.9

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 20.0.0.2  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#standby 2 ip 20.0.0.1

Router(config-if)#standby 2 priority 110

Router(config-if)#standby 2 pre-empt

Router#show standby brief

Router(config-if)#ip dhcp pool DHCP

Router(dhcp-config)#network 20.0.0.0 255.0.0.0

Router(dhcp-config)#default-router 20.0.0.1

Router(dhcp-config)#ip dhcp excluded-address 20.0.0.1 20.0.0.9

Open pc 0 – cmd prompt – type command

>ping 10.0.0.10