Name: Pranali Sonawane

Subject: CCNA

Date:

Lab – HSRP(Hot  standby Router Protocol) with active-backup:

First, we have taken one switch, two PCs, and two routers. The PCs are connected to the switch using straight-through cables. The routers are also connected to the switch using straight-through cables.

Open Switch 0 – CLI prompt then type following commands

Switch>enable

Switch#config terminal

For VLAN 10 :

Switch (config)#vlan 10

Switch (config-vlan)# name CCNA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/1

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#exit

For VLAN 20 :

Switch (config)#vlan 20

Switch (config-vlan)#name MCSA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch(config-if)#exit

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch(config-if)#exit

Switch (config)#interface fastEthernet 0/4

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch(config-if)#exit

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 10.0.0.3 255.0.0.0

Router(config-subif)# exit

Router(config)# interface fastEthernet 0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 20.0.0.3 255.0.0.0

Router(config-subif)# exit

Router(config)# interface fastEthernet 0/0.10

Router(config-subif)#standby 1 ip 10.0.0.10

Router(config-subif)#standby 1 priority 90

Router(config-subif)#standby 1 pre-empt

Router(config-subif)#exit

Router(config)# interface fastEthernet 0/0.20

Router(config-subif)#standby 2 ip 20.0.0.10

Router(config-subif)#standby 2 priority 120

Router(config-subif)#standby 2 pre-empt

Router(config-subif)#exit

Router(config-if)#ip dhcp pool VLAN10_R2

Router(dhcp-config)#network 10.0.0.0 255.0.0.0

Router(dhcp-config)#default-router 10.0.0.10

Router(dhcp-config)#exit

Router(config-if)#ip dhcp pool VLAN20_R2

Router(dhcp-config)#network 20.0.0.0 255.0.0.0

Router(dhcp-config)#default-router 20.0.0.10

Router(dhcp-config)#exit

Router(config)# interface fastEthernet 0/0

Router(config-if)#no sh

Router(config)# exit

Router# show standby brief

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 10.0.0.4 255.0.0.0

Router(config-subif)#standby 1 ip 10.0.0.10

Router(config-subif)#standby 1 priority 120

Router(config-subif)#standby 1 pre-empt

Router(config-subif)#exit

Router(config)# interface fastEthernet 0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 20.0.0.3 255.0.0.0

Router(config-subif)#standby 2 ip 20.0.0.10

Router(config-subif)#standby 2 priority 90

Router(config-subif)#standby 2 pre-empt

Router(config-subif)#exit

Router(config)#ip dhcp pool VLAN10_R1

Router(dhcp-config)#network 10.0.0.0 255.0.0.0

Router(dhcp-config)#default-router 10.0.0.10

Router(dhcp-config)#exit

Router(config)#ip dhcp pool VLAN20_R1

Router(dhcp-config)#network 20.0.0.0 255.0.0.0

Router(dhcp-config)#default-router 20.0.0.10

Router(dhcp-config)#exit

Router(config)# interface fastEthernet 0/0

Router(config-if)#no sh

Router(config)# exit

Router# show standby brief