Name: Pranali Sonawane

Subject: CCNA

Lab – Backup-config :

First, we have taken one  Routers and one server . The Router are connected to the server using Crossover cables.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# hostname ccna

ccna(config)# interface gig 0/0/0

ccna(config-if)# ip address 10.0.0.1  255.0.0.0

ccna(config-if)#no shutdown

ccna(config-if)#exit

ccna(config)#exit

ccna# exit

ccna# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

ccna# copy startup-config tftp

Address or name of remote host []? 10.0.0.2

Destination filename [router-config]? new-router

Writing startup-config...!!

195 bytes copied in 6.2 secs (32 bytes/sec)

ccna# erase startup-config

Erasing the nvram filesystem will remove all configuration files! Continue? [confirm]

[OK]

Erase of nvram: complete

ccna# reload

Proceed with reload? [confirm]

Open server 0 –

  - IPv4 Address: 10.0.0.2

  - Subnet Mask: 255.0.0.0

After go the Server0 ---- services ---- TFTP ----new router is present.

Next : Router 0 is deleted and add the new Router 1.

Router>enable

Router#configure terminal

Router(config)# interface gig 0/0/0

Router(config-if)# ip address 10.0.0.5  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#exit

Router# exit

Router# copy tftp:  startup-config

Address or name of remote host []? 10.0.0.2

Source filename []? new-router

Destination filename [startup-config]?

Loading tftp://10.0.0.2/new-router

Loading new-router from 10.0.0.2: !

[OK - 699 bytes]

699 bytes copied in 3.000 secs (232 bytes/sec)

Router# copy startup-config running-config

Destination filename [running-config]?

1917 bytes copied in 0.416 secs (4606 bytes/sec)

ccna#

Password Recovery

In the same setup, use a backup configuration to perform password recovery.