Name: Pranali Sonawane

Subject: CCNA

Date: 10-11-2025

Lab – Cisco Discovery Protocol(CDP):

First, we have taken five multilayer switches. The switches are connected using crossover cables. The first multilayer switch is connected to five PCs and configured with DHCP.

Open Switch 0 – CLI prompt then type following commands

Switch>enable

Switch#config terminal

Switch (config)#cdp run

Switch (config)#ip routing

Switch (config)#vlan 10

Switch (config-vlan)# name CCNA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/1

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#no cdp enable

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#interface fastEthernet 0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#no cdp enable

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#no cdp enable

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#interface fastEthernet 0/4

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#no cdp enable

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#interface fastEthernet 0/5

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#no cdp enable

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#interface vlan 10

Switch (config-if)# ip address 10.0.0.1 255.255.255.0

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#interface fa0/6

Switch (config-if)# switchport trunk encapsulation dot1q

Switch (config-if)#switchport mode trunk

Switch (config-if)#switchport trunk allowed vlan 10

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#interface fa0/7

Switch (config-if)# switchport trunk encapsulation dot1q

Switch (config-if)#switchport mode trunk

Switch (config-if)#switchport trunk allowed vlan 10

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#interface fa0/8

Switch (config-if)# switchport trunk encapsulation dot1q

Switch (config-if)#switchport mode trunk

Switch (config-if)#switchport trunk allowed vlan 10

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#interface fa0/9

Switch (config-if)# switchport trunk encapsulation dot1q

Switch (config-if)#switchport mode trunk

Switch (config-if)#switchport trunk allowed vlan 10

Switch (config-if)#no shutdown

Switch (config-if)#exit

switch(config-if)#ip dhcp excluded-address 10.0.0.1 10.0.0.10

switch(dhcp-config)#ip dhcp pool VLAN10_POOL

switch(dhcp-config)#network 10.0.0.0 255.255.255.0

switch(dhcp-config)# default-router 10.0.0.1

switch(dhcp-config)#exit

switch(config)#show cdp neighbors

Open Switch 1- CLI prompt then type following commands

Switch>enable

Switch#config terminal

Switch (config)#cdp run

Switch (config)#interface fa0/1

Switch (config-if)# switchport trunk encapsulation dot1q

Switch (config-if)#switchport mode trunk

Switch (config-if)#switchport trunk allowed vlan 10

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#hostname switch1

switch1(config)#

Open Switch 2- CLI prompt then type following commands

Switch>enable

Switch#config terminal

Switch (config)#cdp run

Switch (config)#interface fa0/1

Switch (config-if)# switchport trunk encapsulation dot1q

Switch (config-if)#switchport mode trunk

Switch (config-if)#switchport trunk allowed vlan 10

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#hostname switch2

Switch2(config)#

Open Switch 3- CLI prompt then type following commands

Switch>enable

Switch#config terminal

Switch (config)#cdp run

Switch (config)#interface fa0/1

Switch (config-if)# switchport trunk encapsulation dot1q

Switch (config-if)#switchport mode trunk

Switch (config-if)#switchport trunk allowed vlan 10

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#hostname switch3

Switch3(config)#

Open Switch 4 – CLI prompt then type following commands

Switch>enable

Switch#config terminal

Switch (config)#cdp run

Switch (config)#interface fa0/1

Switch (config-if)# switchport trunk encapsulation dot1q

Switch (config-if)#switchport mode trunk

Switch (config-if)#switchport trunk allowed vlan 10

Switch (config-if)#no shutdown

Switch (config-if)#exit

Switch (config)#hostname switch4

Switch4(config)#