Name: Pranali Sonawane

Subject: CCNA

Date: 11-11-2025

Lab – Router Information Protocol(RIP):

First, we have taken three routers, three switches, and three PCs.The routers are connected to each other using serial cables.Each router is connected to a switch using a straight-through cable, and each switch is connected to a PC using a straight-through cable.

Configure PCs :

- PC0:

  - IPv4 Address: 10.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.1

- PC1:

  - IPv4 Address: 20.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 20.0.0.1

- PC2:

  - IPv4 Address: 30.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 30.0.0.1

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1 255.0.0.0

Router(config-if)#no sh

Router(config)# interface serial0/1/0

Router(config-if)# ip address 40.0.0.1 255.0.0.0

Router(config-if)#no sh

Router(config)# show ip route

Router(config)#router rip

Router(config-router)#version 2

Router(config-router)#network 10.0.0.0

Router(config-router)#network 40.0.0.0

Router(config-router)#exit

Router(config)#exit

Router(config)# show ip route

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 20.0.0.1 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# interface serial0/1/0

Router(config-if)# ip address 40.0.0.2 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# interface serial0/1/0

Router(config-if)# ip address 50.0.0.1 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# show ip route

Router(config)#router rip

Router(config-router)#version 2

Router(config-router)#network 20.0.0.0

Router(config-router)#network 40.0.0.0

Router(config-router)#network 50.0.0.0

Router(config-router)#exit

Router(config)#exit

Router(config)# show ip route

Open Router 2 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 30.0.0.1 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# interface serial0/1/0

Router(config-if)# ip address 50.0.0.2 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# show ip route

Router(config)#router rip

Router(config-router)#version 2

Router(config-router)#network 30.0.0.0

Router(config-router)#network 50.0.0.0

Router(config-router)#exit

Router(config)#exit

Router(config)# show ip route