Name: Pranali Sonawane

Subject: CCNA

Date: 11-11-2025

Lab – Router Information Protocol(RIP):

First, we have taken three routers, two switches.The routers are connected to each other using crossover cable.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface Gig0/0

Router(config-if)# ip address 10.0.0.1 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# interface Gig0/1

Router(config-if)# ip address 20.0.0.1 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# show ip route

Router(config)#router rip

Router(config-router)#version 2

Router(config-router)#network 10.0.0.0

Router(config-router)#network 20.0.0.0

Router(config-router)#exit

Router(config)#exit

Router(config)# show ip route

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface Gig0/0

Router(config-if)# ip address 10.0.0.2 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# interface Gig0/1

Router(config-if)# ip address 30.0.0.1 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# show ip route

Router(config)#router rip

Router(config-router)#version 2

Router(config-router)#network 30.0.0.0

Router(config-router)#network 10.0.0.0

Router(config-router)#exit

Router(config)#exit

Router(config)# show ip route

Open Router 2 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fa0/0

Router(config-if)# ip address 20.0.0.2 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# interface fa0/1

Router(config-if)# ip address 30.0.0.2 255.0.0.0

Router(config-if)#no sh

Router(config)#exit

Router(config)# show ip route

Router(config)#router rip

Router(config-router)#version 2

Router(config-router)#network 20.0.0.0

Router(config-router)#network 30.0.0.0

Router(config-router)#exit

Router(config)#exit

Router(config)# show ip route