Name: Pranali Sonawane

Subject: CCNA

Date: 13-11-2025

Lab – EIGRP Configuration :

First Take 3 Routers (Ex: 1841), 3 Switches And 6 Pc’s,   Connect Routers, Switches And Pc’s Using Straight Through Cables.

Telnet Config:

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# hostname R1

R1(config)# interface fastEthernet 0/0

R1(config-if)# ip address 10.0.0.1  255.0.0.0

R1(config-if)#no shutdown

R1(config-if)#ip dhcp pool DHCP

R1(dhcp-config)# network 10.0.0.0 255.0.0.0

R1(dhcp-config)#default-router 10.0.0.1

R1(dhcp-config)#exit

R1(config)#interface serial 0/1/0

R1(config-if)#ip address 40.0.0.1 255.0.0.0

R1(config-if)#no shutdown

R1(config-if)#exit

R1(config)#ip route 0.0.0.0 0.0.0.0 40.0.0.2

We have given  ip route using default routing to router 0 using command ip route.

Then we configured remote connection using commands:

R1(config)# line vty 0 15

R1(config-line)# password ccna

R1(config-line)# login

R1(config-line)# exit

R1(config)# enable secret ccnp

R1(config)# exit

R1#config t

R1(config)#router eigrp 1

R1(config-router)#network 10.0.0.0 255.0.0.0

R1(config-router)#network 40.0.0.0 255.0.0.0

R1(config-router)#exit

R1(config)#exit

R1#show ip eigrp neighbors

R1#show ip route

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# hostname R2

R2(config)# interface fastEthernet 0/0

R2(config-if)# ip address 20.0.0.1  255.0.0.0

R2(config-if)#no shutdown

R2(config-if)#ip dhcp pool DHCP

R2(dhcp-config)# network 20.0.0.0 255.0.0.0

R2(dhcp-config)#default-router 20.0.0.1

R2(dhcp-config)#exit

R2(config)#interface serial 0/1/0

R2(config-if)#ip address 40.0.0.2 255.0.0.0

R2(config-if)#no shutdown

R2(config)#interface serial 0/1/1

R2(config-if)#ip address 50.0.0.1 255.0.0.0

R2(config-if)#no shutdown

R2(config-if)#exit

R2(config)#ip route 0.0.0.0 0.0.0.0 40.0.0.1

R2(config)#ip route 0.0.0.0 0.0.0.0 50.0.0.2

We have given  ip route using default routing to router 0 using command ip route.

Then we configured remote connection using commands:

R2(config)# line vty 0 15

R2(config-line)# password ccna

R2(config-line)# login

R2(config-line)# exit

R2(config)# enable secret ccnp

R2(config)# exit

R2#config t

R2(config)#router eigrp 1

R2(config-router)#network 20.0.0.0 255.0.0.0

R2(config-router)#network 40.0.0.0 255.0.0.0

R2(config-router)#network 50.0.0.0 255.0.0.0

R2(config-router)#exit

R2(config)#exit

R2#show ip eigrp neighbors

R2#show ip route

Open Router 3– CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# hostname R3

R3(config)# interface fastEthernet 0/0

R3(config-if)# ip address 30.0.0.1  255.0.0.0

R3(config-if)#no shutdown

R3(config-if)#ip dhcp pool DHCP

R3(dhcp-config)# network 30.0.0.0 255.0.0.0

R3(dhcp-config)#default-router 30.0.0.1

R3(dhcp-config)#exit

R3(config)#interface serial 0/1/0

R3(config-if)#ip address 50.0.0.2 255.0.0.0

R3(config-if)#no shutdown

R3(config-if)#exit

R3(config)#ip route 0.0.0.0 0.0.0.0 50.0.0.1

We have given  ip route using default routing to router 0 using command ip route.

Then we configured remote connection using commands:

R3(config)# line vty 0 15

R3(config-line)# password ccna

R3(config-line)# login

R3(config-line)# exit

R3(config)# enable secret ccnp

R3(config)# exit

R3#config t

R3(config)#router eigrp 1

R3(config-router)#network 30.0.0.0 255.0.0.0

R3(config-router)#network 50.0.0.0 255.0.0.0

R3(config-router)#exit

R3(config)#exit

R3#show ip eigrp neighbors

R3#show ip route

First Take 3 Routers (Ex: 1841), 3 Switches And 6 Pc’s,   Connect Routers, Switches And Pc’s Using Straight Through Cables.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config)#exit

Router(config)#interface serial 0/1/0

Router(config-if)#ip address 40.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config)# exit

Router(config)#router eigrp 1

Router(config-router)#network 10.0.0.0 255.0.0.0

Router(config-router)#network 40.0.0.0 255.0.0.0

Router(config-router)#exit

Router(config)#exit

Router#show ip eigrp neighbors

Router#show ip route

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 20.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config)#exit

Router(config)#interface serial 0/1/0

Router(config-if)#ip address 40.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config)# exit

Router(config)#router eigrp 1

Router(config-router)#network 20.0.0.0 255.0.0.0

Router(config-router)#network 40.0.0.0 255.0.0.0

Router(config-router)#network 50.0.0.0 255.0.0.0

Router(config-router)#exit

Router(config)#exit

Router#show ip eigrp neighbors

Router#show ip route

Open Router 2 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 30.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config)#exit

Router(config)#interface serial 0/1/1

Router(config-if)#ip address 50.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config)# exit

Router(config)#router eigrp 1

Router(config-router)#network 30.0.0.0 255.0.0.0

Router(config-router)#network 50.0.0.0 255.0.0.0

Router(config-router)#exit

Router(config)#exit

Router#show ip eigrp neighbors

Router#show ip route