Name: Pranali Sonawane

Subject: CCNA

Date: 14-11-2025

Lab – EIGRP Configuration:

First, take two routers (e.g., 1841), two switches, and four PCs. Connect the routers, switches, and PCs using straight-through cables. Also, connect the serial interfaces using a straight-through cable and connect the FastEthernet interfaces using crossover cables

Serial cable:

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface Serial0/1/0

Router(config-if)#ip address 30.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# router eigrp 1

Router(config-router)#  network 10.0.0.0 255.0.0.0

Router(config-router)# network 30.0.0.0 255.0.0.0

Router(config-router)# exit

Router#show ip route

Router#show interface serial0/1/0

10^7/(LB+CD)*256

(10000000/1544+2010)*256=2172428

( 6476.05+2010)*256

= 2172428

LB:      Fa-100000 kbps

Serial =1544

CD: se=20000/10=2000

Fa=100/10=10