Name: Pranali Sonawane

Subject: CCNA

Date: 15-11-2025

Lab – EIGRP Configuration(Load Balancing) :

First, take two routers (e.g., 1841), two switches, and four PCs. Connect the routers, switches, and PCs using straight-through cables. Also, connect the serial interfaces using a straight-through cable and connect the FastEthernet interfaces using crossover cables

Serial cable:

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface Serial0/1/0

Router(config-if)#ip address 30.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# router eigrp 1

Router(config-router)#  network 10.0.0.0 255.0.0.0

Router(config-router)# network 30.0.0.0 255.0.0.0

Router(config-router)# exit

Router#show ip route

Router#show interface serial0/1/0

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 20.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface Serial0/1/0

Router(config-if)#ip address 30.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# router eigrp 1

Router(config-router)#  network 20.0.0.0 255.0.0.0

Router(config-router)# network 30.0.0.0 255.0.0.0

Router(config-router)# exit

Router#show ip route

Router#show interface serial0/1/0

FastEthernet(crossover cable):

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/1

Router(config-if)# ip address 40.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# router eigrp 1

Router(config-router)#  network 10.0.0.0 255.0.0.0

Router(config-router)# network 30.0.0.0 255.0.0.0

Router(config-router)# network 40.0.0.0 255.0.0.0

Router(config-router)# exit

Router#show ip route

Router#show interface fa0/1

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/1

Router(config-if)# ip address 40.0.0.2  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# router eigrp 1

Router(config-router)#  network 20.0.0.0 255.0.0.0

Router(config-router)# network 30.0.0.0 255.0.0.0

Router(config-router)# network 40.0.0.0 255.0.0.0

Router(config-router)# exit

Router#show ip route

Router#show interface fa0/1

In load balancing, both routers must have the same bandwidth and delay.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface serial0/1/0

Router(config-if)# bandwidth 1000000

Router(config-if)#exit

Router(config)# interface serial0/1/0

Router(config-if)# delay 1000

Router(config-if)#exit

Router(config)# interface fa0/1

Router(config-if)# delay 1000

Router(config-if)#exit

Router(config)#exit

Router#show ip route

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface serial0/1/0

Router(config-if)# bandwidth 1000000

Router(config-if)#exit

Router(config)# interface serial0/1/0

Router(config-if)# delay 1000

Router(config-if)#exit

Router(config)# interface fa0/1

Router(config-if)# delay 1000

Router(config-if)#exit

Router(config)#exit

Router#show ip route