Name: Pranali Sonawane

Subject: CCNA

Date: 18-11-2025

Lab – OSPF Configuration(Basic) :

First, take two routers (e.g., 1841), two switches, and four PCs. Connect the routers, switches, and PCs using straight-through cables. Also, connect the serial interfaces using a straight-through cable and connect the FastEthernet interfaces using crossover cables

Configure PCs :

- PC0:

  - IPv4 Address: 10.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.1

- PC1:

  - IPv4 Address: 10.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.1

- PC2:

  - IPv4 Address: 20.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 20.0.0.1

- PC3:

  - IPv4 Address: 20.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 20.0.0.1

- PC4:

  - IPv4 Address: 30.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 30.0.0.1

- PC5:

  - IPv4 Address: 30.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 30.0.0.1

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface serial 0/1/0

Router(config-if)#ip address 40.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 60.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router ospf 1

Router(config-router)#network 10.0.0.0  0.255.255.255 area 0

Router(config-router)#network 60.0.0.0  0.255.255.255 area 0

Router(config-router)#network 40.0.0.0  0.255.255.255 area 0

Router(config-router)#exit

Router(config)#exit

Router#show ip route

Router#show ip os nei

Open Router 2 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 20.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface serial 0/1/0

Router(config-if)#ip address 40.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# interface fastEthernet 0/1

Router(config-if)# ip address 60.0.0.2  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface serial 0/1/1

Router(config-if)#ip address 50.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router ospf 1

Router(config-router)#network 20.0.0.0  0.255.255.255 area 0

Router(config-router)#network 40.0.0.0  0.255.255.255 area 0

Router(config-router)#network 50.0.0.0  0.255.255.255 area 0

Router(config-router)#network 60.0.0.0  0.255.255.255 area 0

Router(config-router)#exit

Router(config)#exit

Router#show ip route

Router#show ip os nei

Open Router 3 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 30.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface serial 0/1/0

Router(config-if)#ip address 50.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router ospf 1

Router(config-router)#network 30.0.0.0  0.255.255.255 area 0

Router(config-router)#network 50.0.0.0  0.255.255.255 area 0

Router(config-router)#exit

Router(config)#exit

Router#show ip route

Router#show ip os nei