Name: Pranali Sonawane

Subject: CCNA

Date: 19-11-2025

Lab 27– : ACLs on two routers using Standard ACL and Extended ACL.

First, take two routers (for example, 1841 models), two switches, and four PCs.Connect the routers, switches, and PCs using straight-through cables.Use a crossover cable to connect the FastEthernet interfaces between the two routers.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface fa0/1

Router(config-if)#ip address 30.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# router ospf 1

Router(config-router)#  network 10.0.0.0 0.255.255.255 area 0

Router(config-router)# network 30.0.0.0 0.255.255.255 area 0

Router(config-router)# exit

Router#show ip route

Router#show ip os nei

Router 0 - Standard ACL

Router>enable

Router#configure terminal

Router(config)#access-list 1 deny 10.0.0.0 0.255.255.255

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip access-group 1 in (Ping is not Working )

Router(config-if)# no ip access-group 1 in (Ping is working )

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 20.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface fa0/1

Router(config-if)#ip address 30.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# router ospf 1

Router(config-router)#  network 20.0.0.0 0.255.255.255 area 0

Router(config-router)# network 30.0.0.0 0.255.255.255 area 0

Router(config-router)# exit

Router#show ip route

Router#show ip os nei

Router 0 - Standard ACL

Router>enable

Router#configure terminal

Router(config)#access-list 2 deny 20.0.0.0 0.255.255.255

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip access-group 2 in

Router(config-if)# no ip access-group 2 in

Router 0 - Extended ACL:

Router>enable

Router#configure terminal

Router(config)#access-list 101 deny ip host 10.0.0.2 host 20.0.0.2

Router(config)#access-list 101 permit ip any any

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip access-group 101 in

Router(config-if)# no ip access-group 101 in

Self Study:

Router 1 - Extended ACL:

Router>en

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#access-list 102 deny ip host 20.0.0.2 host 10.0.0.2

Router(config)#access-list 102 permit ip any any

Router(config)#interface fa0/0

Router(config-if)#ip access-group 102 in

Router(config-if)#no ip access-group 102 in