Name: Pranali Sonawane

Subject: CCNA

Date: 19-11-2025

Lab 30– : ACLs on Three routers using Standard ACL and Extended ACL.

First, take three routers (for example, 1841 models), three switches, and six PCs.Connect the routers, switches, and PCs using straight-through cables.Use a straight-through cable to connect the serial interfaces between the 3 routers.

Configure PCs :

- PC0:

  - IPv4 Address: 10.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.1

- PC1:

  - IPv4 Address: 10.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.1

- PC2:

  - IPv4 Address: 20.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 20.0.0.1

- PC3:

  - IPv4 Address: 20.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 20.0.0.1

- PC4:

  - IPv4 Address: 30.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 30.0.0.1

- PC5:

  - IPv4 Address: 30.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 30.0.0.1

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface serial 0/1/0

Router(config-if)#ip address 40.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 60.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router ospf 1

Router(config-router)#network 10.0.0.0  0.255.255.255 area 0

Router(config-router)#network 60.0.0.0  0.255.255.255 area 0

Router(config-router)#network 40.0.0.0  0.255.255.255 area 0

Router(config-router)#exit

Router(config)#exit

Router#show ip route

Router#show ip os nei

Router 0 - Standard ACL

Router>enable

Router#configure terminal

Router(config)#access-list 1 deny 10.0.0.0 0.255.255.255

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip access-group 1 in (Ping is not Working )

Router(config-if)# no ip access-group 1 in (Ping is working )

Router 0 - Extended ACL:

Router>enable

Router#configure terminal

Router(config)#access-list 101 deny ip host 10.0.0.2 host 20.0.0.2

Router(config)#access-list 101 permit ip any any

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip access-group 101 in

Router(config-if)# no ip access-group 101 in

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 20.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface serial 0/1/0

Router(config-if)#ip address 40.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# interface fastEthernet 0/1

Router(config-if)# ip address 60.0.0.2  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface serial 0/1/1

Router(config-if)#ip address 50.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router ospf 1

Router(config-router)#network 20.0.0.0  0.255.255.255 area 0

Router(config-router)#network 40.0.0.0  0.255.255.255 area 0

Router(config-router)#network 50.0.0.0  0.255.255.255 area 0

Router(config-router)#network 60.0.0.0  0.255.255.255 area 0

Router(config-router)#exit

Router(config)#exit

Router#show ip route

Router#show ip os nei

Router 1 - Standard ACL

Router>enable

Router#configure terminal

Router(config)#access-list 2 deny 20.0.0.0 0.255.255.255

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip access-group 2 in

Router(config-if)# no ip access-group 2 in

Self Study:

Router 1 - Extended ACL:

Router>en

Router#config t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#access-list 102 deny ip host 20.0.0.2 host 10.0.0.2

Router(config)#access-list 102 permit ip any any

Router(config)#interface fa0/0

Router(config-if)#ip access-group 102 in

Router(config-if)#no ip access-group 102 in

Open Router 3 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 30.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface serial 0/1/0

Router(config-if)#ip address 50.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router ospf 1

Router(config-router)#network 30.0.0.0  0.255.255.255 area 0

Router(config-router)#network 50.0.0.0  0.255.255.255 area 0

Router(config-router)#exit

Router(config)#exit

Router#show ip route

Router#show ip os nei

Router 2 - Standard ACL

Router>enable

Router#configure terminal

Router(config)#access-list 3 deny 30.0.0.0 0.255.255.255

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip access-group 2 in

Router(config-if)# no ip access-group 2 in

One scnerios PC0 is ping is not unreachable to Router 2 30.0.0.1 and PC0 Telnet is working :

Router0> enable

Router0# configure terminal

Router0(config)# no access-list 101

Router0(config)# access-list 101 deny tcp host 10.0.0.2 host 30.0.0.1 eq 23

Route0(config)# access-list 101 deny ip host 10.0.0.2 host 30.0.0.1

Router0(config)# access-list 101 permit ip any any

Router0(config)# interface Fa0/0

Router0(config-if)# ip access-group 101 out

Router0(config-if)# end