Name : Pranali Sonawane

Subject : CCNA

Static Routing (Static Configuration of Router)

Lab Practical 2:

- Start by placing 2 routers, 2 switches, and 4 PCs on your workspace. Connect the routers with a crossover cable. Then, connect each switch to its router and the PCs to the switches using straight-through cables. Assign IP addresses and configure static routes.

Open Router 0 – CLI prompt then type following commands

Step 1: Configure Router R1

- Open Router 0 and go to the CLI.

- Type the following commands:

Router> enable

Router# configure terminal

Router(config)# hostname R1

R1(config)# interface fastEthernet 0/0

R1(config-if)# ip address 10.0.0.1 255.0.0.0

R1(config-if)# no shutdown

Step 2: Configure PCs connected to R1

- PC0:

  - IPv4 Address: 10.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.1

- PC1:

  - IPv4 Address: 10.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.1

Step 3: Configure the second interface of R1

R1(config-if)# exit

R1(config)# interface fastEthernet 0/1

R1(config-if)# ip address 20.0.0.1 255.0.0.0

R1(config-if)# no shutdown

Step 1: Configure Router R2

- Open Router 1 and access the CLI.

- Enter the following commands:

Router> enable

Router# configure terminal

Router(config)# hostname R2

R2(config)# interface fastEthernet 0/0

R2(config-if)# ip address 30.0.0.1 255.0.0.0

R2(config-if)# no shutdown

Step 2: Configure PCs connected to R2

- PC2:

  - IPv4 Address: 30.0.0.2

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 30.0.0.1

- PC3:

  - IPv4 Address: 30.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 30.0.0.1

Step 3: Configure the second interface of R2

R2(config-if)# exit

R2(config)# interface fastEthernet 0/1

R2(config-if)# ip address 20.0.0.2 255.0.0.0

R2(config-if)# no shutdown

Step 1: Add Static Routes on Routers

- On Router R1 (R1 CLI):

R1(config-if)# exit

R1(config)# ip route 30.0.0.0 255.0.0.0 20.0.0.2

- On Router R2 (R2 CLI):

R2(config-if)# exit

R2(config)# ip route 10.0.0.0 255.0.0.0 20.0.0.1

Step 2: Test Connectivity

- On PC0, open the Command Prompt and type:

> ping 30.0.0.2

- On PC2, open the Command Prompt and type:

> ping 10.0.0.1

- First Take 3 Routers (Ex: 1841), 3 Switches And 6 Pc’s, Connect Routers Using Crossover Cable , Switches And Pc’s Using Straight Through Cables.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Open pc 0 – desktop—ip configuration – provide ipv4 address (10.0.0.2) – subnet mask (255.0.0.0) – default gateway (10.0.0.1)

Open pc 1 – desktop—ip configuration – provide ipv4 address (10.0.0.3) – subnet mask (255.0.0.0) – default gateway (10.0.0.1)

Open Router 0 – CLI prompt then type following commands

Router(config-if)#exit

Router(config)# interface serial0/1/0
Router(config-if)# ip address 40.0.0.1 255.0.0.0
Router(config-if)# no shutdown

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 20.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Open pc 2 – desktop—ip configuration – provide ipv4 address (20.0.0.2) – subnet mask (255.0.0.0) – default gateway (20.0.0.1)

Open pc 3 – desktop—ip configuration – provide ipv4 address (20.0.0.3) – subnet mask (255.0.0.0) – default gateway (20.0.0.1)

Open Router 1 – CLI prompt then type following commands

Router(config-if)#exit

Router(config)# interface serial0/1/0
Router(config-if)# ip address 40.0.0.2 255.0.0.0
Router(config-if)# no shutdown

Router(config-if)#exit

Router(config)# interface serial0/1/1
Router(config-if)# ip address 50.0.0.1 255.0.0.0
Router(config-if)# no shutdown

Open Router 2 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 30.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Open pc 4 – desktop—ip configuration – provide ipv4 address (30.0.0.2) – subnet mask (255.0.0.0) – default gateway (30.0.0.1)

Open pc 5 – desktop—ip configuration – provide ipv4 address (30.0.0.3) – subnet mask (255.0.0.0) – default gateway (30.0.0.1)

Open Router 2 – CLI prompt then type following commands

Router(config-if)#exit

Router(config)# interface serial0/1/0
Router(config-if)# ip address 50.0.0.2 255.0.0.0
Router(config-if)# no shutdown

Open Router 0 – CLI prompt then type following command

Router(config-if)#exit

Router(config)#ip route 20.0.0.0 255.0.0.0 40.0.0.2

Router(config)#ip route 30.0.0.0  255.0.0.0  40.0.0.2

Open Router 1 – CLI prompt then type following command

Roter(config-if)#exit

Router(config)#ip route 10.0.0.0 255.0.0.0 40.0.0.1

Router(config)#ip route 30.0.0.0  255.0.0.0  50.0.0.2

Open Router 2 – CLI prompt then type following command

Roter(config-if)#exit

Router(config)#ip route 10.0.0.0 255.0.0.0 50.0.0.1

Router(config)#ip route 20.0.0.0  255.0.0.0  50.0.0.1

Open pc 0 – cmd prompt – type command

>ping 30.0.0.2

Open pc 2 – cmd prompt – type command

>ping 10.0.0.2

Open pc 4 – cmd prompt – type command

>ping 20.0.0.2

- First Take 4 Routers (Ex: 1841), 4 Switches And Pc’s, Connect Routers Using Crossover Cable , Switches And Pc’s Using Straight Through Cables.

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Open pc 0 – desktop—ip configuration – provide ipv4 address (10.0.0.2) – subnet mask (255.0.0.0) – default gateway (10.0.0.1)

Open pc 1 – desktop—ip configuration – provide ipv4 address (10.0.0.3) – subnet mask (255.0.0.0) – default gateway (10.0.0.1)

Open Router 1 – CLI prompt then type following commands

Router(config-if)#exit

Router(config)# interface serial0/1/0
Router(config-if)# ip address 50.0.0.1 255.0.0.0
Router(config-if)# no shutdown

Open Router 2 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 20.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Open pc 2 – desktop—ip configuration – provide ipv4 address (20.0.0.2) – subnet mask (255.0.0.0) – default gateway (20.0.0.1)

Open pc 3 – desktop—ip configuration – provide ipv4 address (20.0.0.3) – subnet mask (255.0.0.0) – default gateway (20.0.0.1)

Open Router 2 – CLI prompt then type following commands

Router(config-if)#exit

Router(config)# interface serial0/1/0
Router(config-if)# ip address 50.0.0.2 255.0.0.0
Router(config-if)# no shutdown

Router(config-if)#exit

Router(config)# interface serial0/1/1
Router(config-if)# ip address 60.0.0.1 255.0.0.0
Router(config-if)# no shutdown

Open Router 3 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 30.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Open pc 4 – desktop—ip configuration – provide ipv4 address (30.0.0.2) – subnet mask (255.0.0.0) – default gateway (30.0.0.1)

Open pc 5 – desktop—ip configuration – provide ipv4 address (30.0.0.3) – subnet mask (255.0.0.0) – default gateway (30.0.0.1)

Open Router 3 – CLI prompt then type following commands

Router(config-if)#exit

Router(config)# interface serial0/1/0
Router(config-if)# ip address 60.0.0.2 255.0.0.0
Router(config-if)# no shutdown

Router(config-if)#exit

Router(config)# interface serial0/1/1
Router(config-if)# ip address 70.0.0.1 255.0.0.0
Router(config-if)# no shutdown

Open Router 4 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 40.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Open pc 6 – desktop—ip configuration – provide ipv4 address (40.0.0.2) – subnet mask (255.0.0.0) – default gateway (40.0.0.1)

Open pc 7 – desktop—ip configuration – provide ipv4 address (40.0.0.3) – subnet mask (255.0.0.0) – default gateway (40.0.0.1)

Open Router 4 – CLI prompt then type following commands

Router(config-if)#exit

Router(config)# interface serial0/1/0
Router(config-if)# ip address 70.0.0.2 255.0.0.0
Router(config-if)# no shutdown

Open Router 1 – CLI prompt then type following command

Router(config-if)#exit

Router(config)#ip route 20.0.0.0 255.0.0.0 50.0.0.2

Router(config)#ip route 30.0.0.0  255.0.0.0  50.0.0.2

Router(config)#ip route 40.0.0.0  255.0.0.0  50.0.0.2

Router(config)#ip route 60.0.0.0  255.0.0.0  50.0.0.2

Router(config)#ip route 70.0.0.0  255.0.0.0  50.0.0.2

Open Router 2 – CLI prompt then type following command

Router(config-if)#exit

Router(config)#ip route 10.0.0.0 255.0.0.0 50.0.0.1

Router(config)#ip route 30.0.0.0  255.0.0.0  60.0.0.2

Router(config)#ip route 70.0.0.0  255.0.0.0  60.0.0.2

Router(config)#ip route 40.0.0.0  255.0.0.0  60.0.0.2

Open Router 3 – CLI prompt then type following command

Router(config-if)#exit

Router(config)#ip route 10.0.0.0 255.0.0.0 60.0.0.1

Router(config)#ip route 20.0.0.0  255.0.0.0  60.0.0.1

Router(config)#ip route 40.0.0.0  255.0.0.0  70.0.0.2

Router(config)#ip route 50.0.0.0  255.0.0.0  60.0.0.1

Open Router 4 – CLI prompt then type following command

Router(config-if)#exit

Router(config)#ip route 10.0.0.0 255.0.0.0 70.0.0.1

Router(config)#ip route 20.0.0.0  255.0.0.0  70.0.0.1

Router(config)#ip route 30.0.0.0  255.0.0.0  70.0.0.1

Router(config)#ip route 50.0.0.0  255.0.0.0  70.0.0.1

Router(config)#ip route 60.0.0.0  255.0.0.0  70.0.0.1

Open pc 0 – cmd prompt – type command

>ping 20.0.0.2

We will get Reply back (acknowledgement ) from pc’s

This Is How We Have Done Static Routing.