Name: Pranali Sonawane

Subject: CCNA

Lab - Telnet Configuration :

First, we have taken 1 router, 1 switch, and 2 PCs. The PCs were connected using copper straight-through cables, and the router was also connected to the switch using a copper straight-through cable.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# hostname R1

R1(config)# interface fastEthernet 0/0

R1(config-if)# ip address 10.0.0.1  255.0.0.0

R1(config-if)#no shutdown

R1(config-if)#ip dhcp pool DHCP

R1(dhcp-config)# network 10.0.0.0 255.0.0.0

R1(dhcp-config)#default-router 10.0.0.1

R1(dhcp-config)#exit

We have given  ip route using default routing to router 0 using command ip route.

Then we configured remote connection using commands:

R1(config)# line vty 0 15

R1(config-line)# password ccna

R1(config-line)# login

R1(config-line)# exit

R1(config)# enable secret ccnp

- First we have taken 2 Routers, 2 switches and 4 pc’s. Pc; connected using straight through cable and routers are connected using cross over cable.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# hostname R1

R1(config)# interface fastEthernet 0/0

R1(config-if)# ip address 10.0.0.1  255.0.0.0

R1(config-if)#no shutdown

R1(config-if)#ip dhcp pool DHCP

R1(dhcp-config)# network 10.0.0.0 255.0.0.0

R1(dhcp-config)#default-router 10.0.0.1

R1(dhcp-config)#exit

R1(config)#interface fastEthernet 0/1

R1(config-if)#ip address 20.0.0.1 255.0.0.0

R1(config-if)#no shutdown

R1(config-if)#exit

R1(config)#ip route 0.0.0.0 0.0.0.0 20.0.0.2

We have given  ip route using default routing to router 0 using command ip route.

Then we configured remote connection using commands:

R1(config)# line vty 0 15

R1(config-line)# password ccna

R1(config-line)# login

R1(config-line)# exit

R1(config)# enable secret ccnp

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# hostname R2

R2(config)# interface fastEthernet 0/0

R2(config-if)# ip address 30.0.0.1  255.0.0.0

R2(config-if)#no shutdown

R2(config-if)#ip dhcp pool DHCP

R2(dhcp-config)# network 30.0.0.0 255.0.0.0

R2(dhcp-config)#default-router 30.0.0.1

R2(dhcp-config)#exit

R2(config)#interface fastEthernet 0/1

R2(config-if)#ip address 20.0.0.2 255.0.0.0

R2(config-if)#no shutdown

R2(config-if)#exit

R2(config)#ip route 0.0.0.0 0.0.0.0 20.0.0.1

We have given  ip route using default routing to router 0 using command ip route.

Then we configured remote connection using commands:

R2(config)# line vty 0 15

R2(config-line)# password ccna

R2(config-line)# login

R2(config-line)# exit

R2(config)# enable secret ccnp

Then we opened pc0—desktop—cmd and entered command :

C:\telnet 20.0.0.2

It  will connect router and ask password for user access verification enter mentioned password then

R2> enable

Password:

It wil again ask for secret password enter that password now you remotely got access for router0

Then we opened pc2—desktop—cmd and entered command :

C:\telnet 20.0.0.1

It  will connect router and ask password for user access verification enter mentioned password then

R1> enable

Password:

It wil again ask for secret password enter that password now you remotely got access for router 1

- First Take 3 Routers (Ex: 1841), 3 Switches And 6 Pc’s, Connect Routers, Switches And Pc’s Using Straight Through Cables.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# hostname R1

R1(config)# interface fastEthernet 0/0

R1(config-if)# ip address 10.0.0.1  255.0.0.0

R1(config-if)#no shutdown

R1(config-if)#ip dhcp pool DHCP

R1(dhcp-config)# network 10.0.0.0 255.0.0.0

R1(dhcp-config)#default-router 10.0.0.1

R1(dhcp-config)#exit

R1(config)#interface serial 0/1/0

R1(config-if)#ip address 40.0.0.1 255.0.0.0

R1(config-if)#no shutdown

R1(config-if)#exit

R1(config)#ip route 0.0.0.0 0.0.0.0 40.0.0.2

We have given  ip route using default routing to router 0 using command ip route.

Then we configured remote connection using commands:

R1(config)# line vty 0 15

R1(config-line)# password ccna

R1(config-line)# login

R1(config-line)# exit

R1(config)# enable secret ccnp

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# hostname R2

R2(config)# interface fastEthernet 0/0

R2(config-if)# ip address 20.0.0.1  255.0.0.0

R2(config-if)#no shutdown

R2(config-if)#ip dhcp pool DHCP

R2(dhcp-config)# network 20.0.0.0 255.0.0.0

R2(dhcp-config)#default-router 30.0.0.1

R2(dhcp-config)#exit

R2(config)#interface serial 0/1/0

R2(config-if)#ip address 40.0.0.2 255.0.0.0

R2(config-if)#no shutdown

R2(config-if)#exit

R2(config)#interface serial 0/1/1

R2(config-if)#ip address 50.0.0.1 255.0.0.0

R2(config-if)#no shutdown

R2(config-if)#exit

R2(config)#ip route 0.0.0.0 0.0.0.0 40.0.0.1

R2(config)#ip route 0.0.0.0 0.0.0.0 50.0.0.2

We have given  ip route using default routing to router 0 using command ip route.

Then we configured remote connection using commands:

R2(config)# line vty 0 15

R2(config-line)# password ccna

R2(config-line)# login

R2(config-line)# exit

R2(config)# enable secret ccnp

Open Router 2 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# hostname R3

R3(config)# interface fastEthernet 0/0

R3(config-if)# ip address 30.0.0.1  255.0.0.0

R3(config-if)#no shutdown

R3(config-if)#ip dhcp pool DHCP

R3(dhcp-config)# network 30.0.0.0 255.0.0.0

R3(dhcp-config)#default-router 30.0.0.1

R3(dhcp-config)#exit

R3(config)#interface serial 0/1/0

R3(config-if)#ip address 50.0.0.2 255.0.0.0

R3(config-if)#no shutdown

R3(config-if)#exit

R3(config)#ip route 0.0.0.0 0.0.0.0 50.0.0.1

We have given  ip route using default routing to router 0 using command ip route.

Then we configured remote connection using commands:

R3(config)# line vty 0 15

R3(config-line)# password ccna

R3(config-line)# login

R3(config-line)# exit

R3(config)# enable secret ccnp

Then we opened pc0—desktop—cmd and entered command :

C:\telnet 20.0.0.1

It  will connect router and ask password for user access verification enter mentioned password then

R2> enable

Password:

It wil again ask for secret password enter that password now you remotely got access for router0