Name: Pranali Sonawane

Subject: CCNA

Lab – Intra-VLAN :

First we have taken 2 switches and 4 pc’s. Pcs connected using straight through cable and switch are connected using cross over cable.

Configure PCs

- PC0:

  - IPv4 Address: 10.0.0.3

  - Subnet Mask: 255.0.0.0

- PC1:

  - IPv4 Address: 20.0.0.3

  - Subnet Mask: 255.0.0.0

- PC2:

  - IPv4 Address: 10.0.0.4

  - Subnet Mask: 255.0.0.0

- Laptop:

  - IPv4 Address: 20.0.0.4

  - Subnet Mask: 255.0.0.0

Open Switch 0 – CLI prompt then type following commands

Switch>enable

Switch#config terminal

For VLAN 10 :

Switch (config)#vlan 10

Switch (config-vlan)# name CCNA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#exit

For VLAN 20 :

Switch (config)#vlan 20

Switch (config-vlan)#name MCSA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch(config-if)#exit

To make interface common use Trunk :

Switch (config)#interface fastEthernet 0/1

Switch (config-if)#switchport mode trunk

Go to Switch1 and enter following commands

Switch>enable

Switch>config terminal

For VLAN 10 :

Switch (config)#vlan 10

Switch (config-vlan) name CCNA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#exit

For VLAN 20 :

Switch (config)#vlan 20

Switch (config-vlan) name MCSA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch (config-if)#exit

To make interface common use Trunk :

Switch (config)#interface fastEthernet 0/1

Switch (config-if)#switchport mode trunk

Open pc 0 – cmd prompt – type command

>ping 10.0.0.4

- First we have taken 3 switches and 6 pc’s. Pcs connected using straight through cable and switch are connected using cross over cable.

Configure PCs

- PC0:

  - IPv4 Address: 10.0.0.3

  - Subnet Mask: 255.0.0.0

- PC1:

  - IPv4 Address: 20.0.0.3

  - Subnet Mask: 255.0.0.0

- PC2:

  - IPv4 Address: 10.0.0.4

  - Subnet Mask: 255.0.0.0

- Laptop:

  - IPv4 Address: 20.0.0.4

  - Subnet Mask: 255.0.0.0

- PC3:

  - IPv4 Address: 10.0.0.5

  - Subnet Mask: 255.0.0.0

- PC4:

  - IPv4 Address: 20.0.0.5

  - Subnet Mask: 255.0.0.0

Open Switch 0 – CLI prompt then type following commands

Switch>enable

Switch#config terminal

For VLAN 10 :

Switch (config)#vlan 10

Switch (config-vlan)# name CCNA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#exit

For VLAN 20 :

Switch (config)#vlan 20

Switch (config-vlan)#name MCSA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch(config-if)#exit

To make interface common use Trunk :

Switch (config)#interface fastEthernet 0/1

Switch (config-if)#switchport mode trunk

Go to Switch1 and enter following commands

Switch>enable

Switch>config terminal

For VLAN 10 :

Switch (config)#vlan 10

Switch (config-vlan) name CCNA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#exit

For VLAN 20 :

Switch (config)#vlan 20

Switch (config-vlan) name MCSA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch (config-if)#exit

To make interface common use Trunk :

Switch (config)#interface fastEthernet 0/1

Switch (config-if)#switchport mode trunk

Switch (config)#interface fastEthernet 0/4

Switch (config-if)#switchport mode trunk

Go to Switch 2 and enter following commands

Switch>enable

Switch>config terminal

For VLAN 10 :

Switch (config)#vlan 10

Switch (config-vlan) name CCNA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#exit

For VLAN 20 :

Switch (config)#vlan 20

Switch (config-vlan) name MCSA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch (config-if)#exit

To make interface common use Trunk :

Switch (config)#interface fastEthernet 0/1

Switch (config-if)#switchport mode trunk

Open pc 0 – cmd prompt – type command

>ping 10.0.0.4