Name: Pranali Sonawane

Subject: CCNA

Lab – Router on a Stick(ROAS) :

First we have taken 1 Router, 1 switches and 5 pc’s. Pcs connected using straight through cable and switch are connected Router using straight through cable.

Configure PCs

- PC0:

  - IPv4 Address: 10.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.1

- PC1:

  - IPv4 Address: 20.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 20.0.0.1

- PC2:

  - IPv4 Address: 30.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 30.0.0.1

- PC3:

  - IPv4 Address: 40.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 40.0.0.1

- PC4:

  - IPv4 Address: 50.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 50.0.0.1

Open Switch 0 – CLI prompt then type following commands

Switch>enable

Switch#config terminal

For VLAN 10 :

Switch (config)#vlan 10

Switch (config-vlan)# name CCNA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/1

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#exit

For VLAN 20 :

Switch (config)#vlan 20

Switch (config-vlan)#name MCSA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch(config-if)#exit

For VLAN 30 :

Switch (config)#vlan 30

Switch (config-vlan)#name Linux

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 30

Switch(config-if)#exit

For VLAN 40 :

Switch (config)#vlan 40

Switch (config-vlan)#name AWS

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/4

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 40

Switch(config-if)#exit

For VLAN 50 :

Switch (config)#vlan 50

Switch (config-vlan)#name Azure

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/5

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 50

Switch(config-if)#exit

To make interface common use Trunk :

Switch (config)#interface fastEthernet 0/6

Switch (config-if)#switchport mode trunk

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)#no shutdown

Router(config)# exit

Router(config)# interface fastEthernet 0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 10.0.0.1 255.0.0.0
Router(config-subif)# exit

Router(config)# interface fastEthernet 0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 20.0.0.1 255.0.0.0
Router(config-subif)# exit

Router(config)# interface fastEthernet 0/0.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# ip address 30.0.0.1 255.0.0.0
Router(config-subif)# exit

Router(config)# interface fastEthernet 0/0.40

Router(config-subif)# encapsulation dot1Q 40

Router(config-subif)# ip address 40.0.0.1 255.0.0.0
Router(config-subif)# exit

Router(config)# interface fastEthernet 0/0.50

Router(config-subif)# encapsulation dot1Q 50

Router(config-subif)# ip address 50.0.0.1 255.0.0.0
Router(config-subif)# exit

First we have taken 1 Router, 2 switches and 4 pc’s. Pcs connected Switch using straight through cable and switch are connected Router using straight through cable.

Configure PCs

- PC0:

  - IPv4 Address: 10.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 10.0.0.1

- PC1:

  - IPv4 Address: 20.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 20.0.0.1

- PC2:

  - IPv4 Address: 30.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 30.0.0.1

- PC3:

  - IPv4 Address: 40.0.0.3

  - Subnet Mask: 255.0.0.0

  - Default Gateway: 40.0.0.1

Open Switch 0 – CLI prompt then type following commands

Switch>enable

Switch#config terminal

For VLAN 10 :

Switch (config)#vlan 10

Switch (config-vlan)# name CCNA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/1

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 10

Switch (config-if)#exit

For VLAN 20 :

Switch (config)#vlan 20

Switch (config-vlan)#name MCSA

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 20

Switch(config-if)#exit

To make interface common use Trunk :

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode trunk

Open Switch 1 – CLI prompt then type following commands

For VLAN 30 :

Switch (config)#vlan 30

Switch (config-vlan)#name Linux

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 30

Switch(config-if)#exit

For VLAN 40 :

Switch (config)#vlan 40

Switch (config-vlan)#name AWS

Switch (config-vlan)#exit

Switch (config)#interface fastEthernet 0/4

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 40

Switch(config-if)#exit

To make interface common use Trunk :

Switch (config)#interface fastEthernet 0/3

Switch (config-if)#switchport mode trunk

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)#no shutdown

Router(config)# exit

Router(config)# interface fastEthernet 0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 10.0.0.1 255.0.0.0
Router(config-subif)# exit

Router(config)# interface fastEthernet 0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 20.0.0.1 255.0.0.0
Router(config-subif)# exit

Router(config)# interface fastEthernet 0/1.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# ip address 30.0.0.1 255.0.0.0
Router(config-subif)# exit

Router(config)# interface fastEthernet 0/1.40

Router(config-subif)# encapsulation dot1Q 40

Router(config-subif)# ip address 40.0.0.1 255.0.0.0
Router(config-subif)# exit