Name: Pranali Sonawane

Subject: CCNA

Lab – Inter-VLAN routing using SVI (Switch Virtual Interface):

First, we have taken one switch and five PCs. The PCs are connected to the switch using straight-through cables. We used a Multilayer Switch 0 (3560-24PS) because it is a Layer 3 switch.

Open Switch 0(3560-24PS) – CLI prompt then type following commands

Switch>enable

Switch#config terminal

Switch (config)#vlan 10

Switch (config-vlan)# name Sales

Switch (config-vlan)# vlan 20

Switch (config-vlan)# name marketing

Switch (config-vlan)# vlan 30

Switch (config-vlan)# name HR

Switch (config-vlan)# vlan 40

Switch (config-vlan)# name Tech

Switch (config-vlan)# vlan 50

Switch (config-vlan)# name Non-Tech

Switch (config-vlan)# interface fastEthernet0/1

Switch (config-if)# switchport mode access

Switch (config-if)# switchport  access vlan 10

Switch (config-if)# exit

Switch (config-vlan)# interface fastEthernet0/2

Switch (config-if)# switchport mode access

Switch (config-if)# switchport  access vlan 20

Switch (config-if)# exit

Switch (config-vlan)# interface fastEthernet0/3

Switch (config-if)# switchport mode access

Switch (config-if)# switchport  access vlan 30

Switch (config-if)# exit

Switch (config-vlan)# interface fastEthernet0/4

Switch (config-if)# switchport mode access

Switch (config-if)# switchport  access vlan 40

Switch (config-if)# exit

Switch (config-vlan)# interface fastEthernet0/5

Switch (config-if)# switchport mode access

Switch (config-if)# switchport  access vlan 50

Switch (config)# exit

Switch (config)# interface vlan 10

Switch (config-if)# ip address 10.0.0.1 255.0.0.0

Switch (config-if)# exit

Switch (config)# interface vlan 20

Switch (config-if)# ip address 20.0.0.1 255.0.0.0

Switch (config-if)# exit

Switch (config)# interface vlan 30

Switch (config-if)# ip address 30.0.0.1 255.0.0.0

Switch (config-if)# exit

Switch (config)# interface vlan 40

Switch (config-if)# ip address 40.0.0.1 255.0.0.0

Switch (config-if)# exit

Switch (config)# interface vlan 50

Switch (config-if)# ip address 50.0.0.1 255.0.0.0

Switch (config-if)# exit

Switch (config)# ip routing

Switch(config)# ip routing

ip routing → This command enables routing on the Layer 3 switch so it can route between VLANs (Inter-VLAN routing).