Name: Pranali Sonawane

Subject: CCNA

Date: 20-11-2025

Lab 31– : NAT -PAT

First, take two routers (for example, 1841 models), two switches, and four PCs.Connect the routers, switches, and PCs using straight-through cables.Use a crossover cable to connect the FastEthernet interfaces between the two routers.

Open Router 0 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 10.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface fa0/1

Router(config-if)#ip address 12.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# router ospf 1

Router(config-router)#  network 10.0.0.0 0.255.255.255 area 0

Router(config-router)# network 12.0.0.0 0.255.255.255 area 0

Router(config-router)# exit

Router#show ip route

Router#show ip os nei

Open Router 1 – CLI prompt then type following commands

Router>enable

Router#configure terminal

Router(config)# interface fastEthernet 0/0

Router(config-if)# ip address 20.0.0.1  255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface fa0/1

Router(config-if)#ip address 12.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# router ospf 1

Router(config-router)#  network 20.0.0.0 0.255.255.255 area 0

Router(config-router)# network 12.0.0.0 0.255.255.255 area 0

Router(config-router)# exit

Router#show ip route

Router#show ip os nei

NAT-PAT:

Open Router 0 – CLI prompt then type following commands

Router >en

Router#config t

Router(config )#access-list 3 permit 20.0.0.0  0.255.255.255

Router(config)#ip nat pool ccna 12.0.0.10 12.0.0.10 netmask 255.0.0.0

Router(Config)# ip nat inside source list 3 pool ccna

Router(config)int fa0/0

Router(config-if)#ip nat inside

Router(config-if)#exit

Router(config)int fa0/1

Router(config-if)#ip nat outside

Router(config-if)#exit

Router# show ip nat translation